COSHH ASSESSMENT SHEET

| # Substance | # DERV |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Description and Application | It is a clear liquid. It is a diesel fuel used for road vehicles. |  |  |  |  |
| # Hazards | The substance is hazardous; it reacts violently with oxidisers, with a risk of irreversible effects. |  |  |  |  |
| Precautions & Control Measures | Use impervious gloves, and wash exposed skin areas several times daily. Use supplied air respiratory protection for clearing large spills or upon entry into large tanks or vessels. |  |  |  |  |
| # Storage | Store in a dry weatherproof tank, inside an impervious bund of adequate size. |  |  |  |  |
| # Spillage & Cleaning | Do not allow spillage to enter a watercourse or a surface water drainage system. In case of spillage on water, contain using a boom and collect by skimming or absorption. For other spillages, contain and absorb in dry sand or earth and seek disposal advice. |  |  |  |  |
| # First Aid |  | Inhalation: |  | Remove to fresh air. If not breathing, give artificial respiration. Seek medical attention. |  |
|  |  | Skin Contact: |  | Wash skin with soap and water. |  |
|  |  | Eye Contact: |  | Flush with plenty of water. |  |
|  |  | Ingestion: |  | Seek medical attention immediately. DO NOT INDUCE VOMITING. |  |
|  |  | Pressure Injection: |  | ALWAYS seek immediate medical attention even if the injury appears minor. |  |
| Health Surveillance | Occasional use only. Health surveillance is not necessary. |  |  |  |  |
| # Fire |  | Flammability: |  |  | Combustible. |
|  |  | Products of Combustion: |  |  | Oxides may be formed on burning in limited air supply. |
|  |  | Extinguishing Media: |  |  | Use foam, dry powder, or CO2. |
| H&S Data Sheet Location |  |  | Manager’s office. |  |  |

COSHH ASSESSMENT

The first page should be printed, laminated and posted where the product is held. The information below forms part of the overall COSHH assessment for the product.

| Is it possible to avoid the need to use hazardous substances? | Yes | No |
| --- | --- | --- |
|  |  | X |

| Duration of exposure (please state in minutes per day) |  |
| --- | --- |

| Persons at risk: | Employees | Students | Others | Vulnerable persons |
| --- | --- | --- | --- | --- |

| Which for does the substance take? | Gas | Vapour |  | Mist | Fume |  | Dust |  | Liquid | Solid |  | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | For other please state: |  |  |  |  |  |  |  |  |  |  |  |
| What is the route of exposure? | Inhalation |  | Skin |  |  | Eyes |  | Ingestion |  |  | Other |  |
|  | For other please state: |  |  |  |  |  |  |  |  |  |  |  |

| Does a Workplace Exposure Limit (WELs) apply to this product? |  |  |  | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Long-term exposure level (8hrTWA): |  | Short-term exposure level (15 mins): |  |  |  |
| Control Measures: List below control measures eg extraction, ventilation, supervision, include additonal controls for vulnerable persons where necessary |  |  |  |  |  |
| Is exposure adequately controlled? |  |  |  | Yes | No |
| Signature | Date |  | Review date |  |  |